How to use the VR Headset / Steps 1 – 4

| 1) Check the equipment | 2) Choose an experience |
| --- | --- |
| Each kit includes the following: Samsung GearVR headset w/ velcro straps, Samsung Galaxy S7 Smartphone, USB Micro charging cable, USB-C charging cable, disposable/reusable liners. | A) Select an experience from the menu and press the orange play button. B) Make sure you see the start screen that says, “please insert your device into your GearVR.” |
| 3) Open the clips | 4) Insert the device |
| Open the two clips before inserting the phone. The USB clip has two positions, A and B. Make sure it is locked in position B. | Plug in the headphones to the auxiliary port, before inserting the phone into the headset. Push firmly to make a tight connection between the USB plug on the headset and the USB port on the phone. Then, snap the phone into place using the clips. |

How to use the VR Headset / Steps 5 - 8

| 5) Apply bouffant cap | 6) Apply disposable liners |
| --- | --- |
| Fit the bouffant cap so that it is covering the patient’s hair. | Fit the Velcro disposable liner to the VR headset |
| 7) Patient begins experience | 8) Patient adjusts settings/controls in VR |
| Help the patient put on the VR headset by adjusting the straps and handing them the headphones. After they put on the headset, the experience will automatically begin. | A) Open the VR menu by holding your gaze at the gear icon. B) Use the controls on the menu to select a new experience, recenter the visuals, exit the menu, or adjust the volume. |